Project Gutenberg's Instruction for Using a Slide Rule, by W. Stanley

This eBook is for the use of anyone anywhere at no cost and with

almost no restrictions whatsoever.  You may copy it, give it away or

re-use it under the terms of the Project Gutenberg License included

with this eBook or online at www.gutenberg.org

Title: Instruction for Using a Slide Rule

Author: W. Stanley

Release Date: December 29, 2006 [EBook #20214]

Language: English

Character set encoding: ASCII

*** START OF THIS PROJECT GUTENBERG EBOOK INSTRUCTION FOR USING A SLIDE RULE ***

Produced by Don Kostuch

[Transcriber's Notes]

Conventional mathematical notation requires specialized fonts and typesetting conventions. I have adopted modern computer programming notation using only ASCII characters. The square root of 9 is thus rendered as square_root(9) and the square of 9 is  square(9) . 10 divided by 5 is (10/5) and 10 multiplied by 5 is (10 * 5 ).

The DOC file and TXT files otherwise closely approximate the original text. There are two versions of the HTML files, one closely approximating the original, and a second with images of the slide rule settings for each example.

By the time I finished engineering school in 1963, the slide rule was a well worn tool of my trade. I did not use an electronic calculator for another ten years. Consider that my predecessors had little else to use--think Boulder Dam (with all its electrical, mechanical and construction calculations).

Rather than dealing with elaborate rules for positioning the decimal point, I was taught to first "scale" the factors and deal with the decimal position separately. For example:

1230 * .000093 =

1.23E3 * 9.3E-5

1.23E3 means multiply 1.23 by 10 to the power 3.

9.3E-5 means multiply 9.3 by 0.1 to the power 5 or 10 to the power -5.

The computation is thus

1.23 * 9.3 * 1E3 * 1E-5

The exponents are simply added.

1.23 *  9.3 * 1E-2 =

11.4 * 1E-2 =

.114

When taking roots, divide the exponent by the root. The square root of 1E6 is 1E3.

The cube root of 1E12 is 1E4.

When taking powers, multiply the exponent by the power.

The cube of 1E5 is 1E15.

[End Transcriber's Notes]

INSTRUCTIONS

for using a

SLIDE

RULE

SAVE TIME!

DO THE FOLLOWING INSTANTLY WITHOUT PAPER AND PENCIL

MULTIPLICATION

DIVISION

RECIPROCAL VALUES

SQUARES & CUBES

EXTRACTION OF SQUARE ROOT

EXTRACTION OF CUBE ROOT

DIAMETER OR AREA OF CIRCLE

SLIDE RULE INSTRUCTIONS 	1

INSTRUCTIONS FOR USING A SLIDE RULE

The slide rule is a device for easily and quickly multiplying, dividing and extracting square root and cube root. It will also perform any combination of these processes. On this account, it is found extremely useful by students and teachers in schools and colleges, by engineers, architects, draftsmen, surveyors, chemists, and many others. Accountants and clerks find it very helpful when approximate calculations must be made rapidly. The operation of a slide rule is extremely easy, and it is well worth while for anyone who is called upon to do much numerical calculation to learn to use one. It is the purpose of this manual to explain the operation in such a way that a person who has never before used a slide rule may teach himself to do so.

DESCRIPTION OF SLIDE RULE

The slide rule consists of three parts (see figure 1). B is the body of the rule and carries three scales marked A, D and K. S is the slider which moves relative to the body and also carries three scales marked B, CI and C. R is the runner or indicator and is marked in the center with a hair-line. The scales A and B are identical and are used in problems involving square root. Scales C and D are also identical and are used for multiplication and division. Scale K is for finding cube root. Scale CI, or C-inverse, is like scale C except that it is laid off from right to left instead of from left to right. It is useful in problems involving reciprocals.

MULTIPLICATION

We will start with a very simple example:

Example 1:  2 * 3 = 6

2 	SLIDE RULE INSTRUCTIONS

To prove this on the slide rule, move the slider so that the 1 at the left-hand end of the C scale is directly over the large 2 on the D scale (see figure 1). Then move the runner till the hair-line is over 3 on the C scale. Read the answer, 6, on the D scale under the hair-line. Now, let us consider a more complicated example:

Example 2:   2.12 * 3.16 = 6.70

As before, set the 1 at the left-hand end of the C scale, which we will call the left- hand index of the C scale, over 2.12 on the D scale (See figure 2). The hair-line of the runner is now placed over 3.16 on the C scale and the answer, 6.70, read on the D scale.

METHOD OF MAKING SETTINGS

In order to understand just why 2.12 is set where it is (figure 2), notice that the interval from 2 to 3 is divided into 10 large or major divisions, each of which is, of course, equal to one-tenth (0.1) of the amount represented by the whole interval. The major divisions are in turn divided into 5 small or minor divisions, each of which is one-fifth or two-tenths (0.2) of the major division, that is 0.02 of the whole interval. Therefore, the index is set above

2 + 1 major division + 1 minor division = 2 + 0.1 + 0.02 = 2.12.

In the same way we find 3.16 on the C scale.

While we are on this subject, notice that in the interval from 1 to 2 the major divisions are marked with the small figures 1 to 9 and the minor divisions are 0.1 of the major divisions. In the intervals from 2 to 3 and 3 to 4 the minor divisions are 0.2 of the major divisions, and for the rest of the D (or C) scale, the minor divisions are 0.5 of the major divisions.

Reading the setting from a slide rule is very much like reading measurements from a ruler. Imagine that the divisions between 2 and 3 on the D scale (figure 2) are those of a ruler divided into tenths of a foot, and each tenth of a foot divided in 5 parts 0.02 of a foot long.

SLIDE RULE INSTRUCTIONS 	3

Then the distance from one on the left-hand end of the D scale (not shown in figure 2) to one on the left-hand end of the C scale would he 2.12 feet. Of course, a foot rule is divided into parts of uniform length, while those on a slide rule get smaller toward the right-hand end, but this example may help to give an idea of the method of making and reading settings. Now consider another example.

Example 3a:  2.12 * 7.35 = 15.6

If we set the left-hand index of the C scale over 2.12 as in the last example, we find that 7.35 on the C scale falls out beyond the body of the rule. In a case like this, simply use the right-hand index of the C scale. If we set this over 2.12 on the D scale and move the runner to 7.35 on the C scale we read the result 15.6 on the D scale under the hair-line.

Now, the question immediately arises, why did we call the result 15.6 and not 1.56? The answer is that the slide rule takes no account of decimal points. Thus, the settings would be identical for all of the following products:

Example 3:

a-- 2.12 * 7.35 = 15.6

b-- 21.2 * 7.35 = 156.0

c-- 212 * 73.5 = 15600

d-- 2.12 * .0735 = .156

e-- .00212 * 735 = .0156

The most convenient way to locate the decimal point is to make a mental multiplication using only the first digits in the given factors. Then place the decimal point in the slide rule result so that its value is nearest that of the mental multiplication. Thus, in example 3a above, we can multiply 2 by 7 in our heads and see immediately that the decimal point must be placed in the slide rule result 156 so that it becomes 15.6 which is nearest to 14. In example 3b (20 * 7 = 140), so we must place the decimal point to give 156. The reader can readily verify the other examples in the same way.

4	SLIDE RULE INSTRUCTIONS

Since the product of a number by a second number is the same as the product of the second by the first, it makes no difference which of the two numbers is set first on the slide rule. Thus, an alternative way of working example 2 would be to set the left-hand index of the C scale over 3.16 on the D scale and move the runner to 2.12 on the C scale and read the answer under the hair-line on the D scale.

The A and B scales are made up of two identical halves each of which is very similar to the C and D scales. Multiplication can also be carried out on either half of the A and B scales exactly as it is done on the C and D scales. However, since the A and B scales are only half as long as the C and D scales, the accuracy is not as good. It is sometimes convenient to multiply on the A and B scales in more complicated problems as we shall see later on.

A group of examples follow which cover all the possible combination of settings which can arise in the multiplication of two numbers.

Example

4: 20 * 3 = 60

5: 85 * 2 = 170

6: 45 * 35 = 1575

7: 151 * 42 = 6342

8: 6.5 * 15 = 97.5

9: .34 * .08 = .0272

10: 75 * 26 = 1950

11: .00054 * 1.4 = .000756

12: 11.1 * 2.7 = 29.97

13: 1.01 * 54 = 54.5

14: 3.14 * 25 = 78.5

SLIDE RULE INSTRUCTIONS 	5

DIVISION

Since multiplication and division are inverse processes, division on a slide rule is done by making the same settings as for multiplication, but in reverse order. Suppose we have the example:

Example 15: (6.70 / 2.12) = 3.16

Set indicator over the dividend 6.70 on the D scale. Move the slider until the divisor 2.12 on the C scale is under the hair-line. Then read the result on the D scale under the left-hand index of the C scale. As in multiplication, the decimal point must be placed by a separate process. Make all the digits except the first in both dividend and divisor equal zero and mentally divide the resulting numbers. Place the decimal point in the slide rule result so that it is nearest to the mental result. In example 15, we mentally divide 6 by 2. Then we place the decimal point in the slide rule result 316 so that it is 3.16 which is nearest to 3.

A group of examples for practice in division follow:

Example

16: 34 / 2 = 17

17: 49 / 7 = 7

18: 132 / 12 = 11

19: 480 / 16 = 30

20: 1.05 / 35 = .03

21: 4.32 / 12 = .36

22: 5.23 / 6.15 = .85

23: 17.1 / 4.5 = 3.8

24: 1895 / 6.06 = 313

25: 45 / .017 = 2647

THE CI SCALE

If we divide one (1) by any number the answer is called the reciprocal of the number. Thus, one-half is the reciprocal of two, one-quarter is the reciprocal of four.

6 	SLIDE RULE INSTRUCTIONS

If we take any number, say 14, and multiply it by the reciprocal of another number, say 2, we get:

Example 26: 14 * (1/2) = 7

which is the same as 14 divided by two. This process can be carried out directly on the slide rule by use of the CI scale. Numbers on the CI scale are reciprocals of those on the C scale. Thus we see that 2 on the CI scale comes directly over 0.5 or 1/2 on the C scale. Similarly 4 on the CI scale comes over 0.25 or 1/4 on the C scale, and so on. To do example 26 by use of the CI scale, proceed exactly as if you were going to multiply in the usual manner except that you use the CI scale instead of the C scale. First set the left-hand index of the C scale over 14 on the D scale. Then move the indicator to 2 on the CI scale. Read the result, 7, on the D scale under the hair-line. This is really another way of dividing. THE READER IS ADVISED TO WORK EXAMPLES 16 TO 25 OVER AGAIN BY USE OF THE CI SCALE.

SQUARING AND SQUARE ROOT

If we take a number and multiply it by itself we call the result the square of the number. The process is called squaring the number. If we find the number which, when multiplied by itself is equal to a given number, the former number is called the square root of the given number. The process is called extracting the square root of the number. Both these processes may be carried out on the A and D scales of a slide rule. For example:

Example 27: 4 * 4 = square( 4 ) = 16

Set indicator over 4 on D scale. Read 16 on A scale under hair-line.

Example 28: square( 25.4 )  = 646.0

SLIDE RULE INSTRUCTIONS 	7

The decimal point must be placed by mental survey. We know that square( 25.4  ) must be a little larger than square( 25 ) = 625 so that it must be 646.0.

To extract a square root, we set the indicator over the number on the A scale and read the result under the hair-line on the D scale. When we examine the A scale we see that there are two places where any given number may be set, so we must have some way of deciding in a given case which half of the A scale to use. The rule is as follows:

(a) If the number is greater than one. For an odd number of digits to the left of the decimal point, use the left-hand half of the A scale. For an even number of digits to the left of the decimal point, use the right-hand half of the A scale.

(b) If the number is less than one. For an odd number of zeros to the right of the decimal point before the first digit not a zero, use the left-hand half of the A scale. For none or any even number of zeros to the right of the decimal point before the first digit not a zero, use the right-hand half of the A scale.

Example 29: square_root( 157 ) = 12.5

Since we have an odd number of digits set indicator over 157 on left-hand half of A scale. Read 12.5 on the D scale under hair-line. To check the decimal point think of the perfect square nearest to 157. It is

12 * 12 = 144, so that square_root(157) must be a little more than 12 or 12.5.

Example 30: square_root( .0037 ) = .0608

In this number we have an even number of zeros to the right of the decimal point, so we must set the indicator over 37 on the right-hand half of the A scale. Read 608 under the hair-line on D scale.

8 	SLIDE RULE INSTRUCTIONS

To place the decimal point write:

square_root( .0037 ) = square_root( 37/10000 )

= 1/100 square_root( 37 )

The nearest perfect square to 37 is 6 * 6 = 36, so the answer should be a little more than 0.06 or .0608. All of what has been said about use of the A and D scales for squaring and extracting square root applies equally well to the B and C scales since they are identical to the A and D scales respectively.

A number of examples follow for squaring and the extraction of square root.

Example

31: square( 2 ) = 4

32: square( 15 ) = 225

33: square( 26 )  = 676

34: square( 19.65 ) = 386

35: square_root( 64 ) = 8

36: square_root( 6.4 ) = 2.53

37: square_root( 498 ) = 22.5

38: square_root( 2500 ) = 50

39: square_root( .16 ) = .04

40: square_root( .03 ) = .173

CUBING AND CUBE ROOT

If we take a number and multiply it by itself, and then multiply the result by the original number we get what is called the cube of the original number. This process is called cubing the number. The reverse process of finding the number which, when multiplied by itself and then by itself again, is equal to the given number, is called extracting the cube root of the given number. Thus, since 5 * 5 * 5 = 125, 125 is the cube of 5 and 5 is the cube root of 125.

SLIDE RULE INSTRUCTIONS 	9

To find the cube of any number on the slide rule set the indicator over the number on the D scale and read the answer on the K scale under the hair-line. To find the cube root of any number set the indicator over the number on the K scale and read the answer on the D scale under the hair-line. Just as on the A scale, where there were two places where you could set a given number, on the K scale there are three places where a number may be set. To tell which of the three to use, we must make use of the following rule.

(a) If the number is greater than one. For 1, 4, 7, 10, etc., digits to the left of the decimal point, use the left-hand third of the K scale. For 2, 5, 8, 11, etc., digits to the left of the decimal point, use the middle third of the K scale. For 3, 6, 9, 12, etc., digits to the left of the decimal point use the right-hand third of the K scale.

(b) If the number is less than one. We now tell which scale to use by counting the number of zeros to the right of the decimal point before the first digit not zero. If there are 2, 5, 8, 11, etc., zeros, use the left-hand third of the K scale. If there are 1, 4, 7, 10, etc., zeros, then use the middle third of the K scale. If there are no zeros or 3, 6, 9, 12, etc., zeros, then use the right-hand third of the K scale. For example:

Example 41: cube_root( 85 ) = 5.70

Since there are 3 digits in the given number, we set the indicator on 185 in the right-hand third of the K scale, and read the result 570 on the D scale. We can place the decimal point by thinking of the nearest perfect cube, which is 125. Therefore, the decimal point must be placed so as to give 5.70, which is nearest to 5, the cube root of 125.

Example 42: cube_root( .034 ) = .324

Since there is one zero between the decimal point and the first digit not zero, we must set the indicator over 34 on the middle third of the K scale.

10 	SLIDE RULE INSTRUCTIONS

We read the result 324 on the D scale. The decimal point may be placed as follows:

cube_root( .034 ) = cube_root( 34/1000  )

= 1/10 cube_root( 34 )

The nearest perfect cube to 34 is 27, so our answer must be close to one-tenth of the cube root of 27 or nearly 0.3. Therefore, we must place the decimal point to give 0.324. A group of examples for practice in extraction of cube root follows:

Example

43: cube_root( 64 ) = 4

44: cube_root( 8 ) = 2

45: cube_root( 343 ) = 7

46: cube_root( .000715 ) = .0894

47: cube_root( .00715 ) = .193

48: cube_root( .0715 ) = .415

49: cube_root( .516 ) = .803

50: cube_root( 27.8 ) = 3.03

51: cube_root( 5.49 ) = 1.76

52: cube_root( 87.1 ) = 4.43

THE 1.5 AND 2/3 POWER

If the indicator is set over a given number on the A scale, the number under the hair-line on the K scale is the 1.5 power of the given number. If the indicator is set over a given number on the K scale, the number under the hair-line on the A scale is the 2/3 power of the given number.

COMBINATIONS OF PROCESSES

A slide rule is especially useful where some combination of processes is necessary, like multiplying 3 numbers together and dividing by a third. Operations of this sort may be performed in such a way that the final answer is obtained immediately without finding intermediate results.

SLIDE RULE INSTRUCTIONS 	11

1. Multiplying several numbers together. For example, suppose it is desired to multiply 4 * 8 * 6. Place the right-hand index of the C scale over 4 on the D scale and set the indicator over 8 on the C scale. Now, leaving the indicator where it is, move the slider till the right-hand index is under the hairline. Now, leaving the slider where it is, move the indicator until it is over 6 on the C scale, and read the result, 192, on the D scale. This may be continued indefinitely, and so as many numbers as desired may be multiplied together.

Example 53: 2.32 * 154 * .0375 * .56 = 7.54

2. Multiplication and division. Suppose we wish to do the following example:

Example 54:  (4 * 15) / 2.5  = 24

First divide 4 by 2.5. Set indicator over 4 on the D scale and move the slider until 2.5 is under the hair-line. The result of this division, 1.6, appears under the left-hand index of the C scale. We do not need to write it down, however, but we can immediately move the indicator to 15 on the C scale and read the final result 24 on the D scale under the hair-line. Let us consider a more complicated problem of the same type:

Example 55: (30/7.5) * (2/4) * (4.5/5) * (1.5/3) = .9

First set indicator over 30 on the D scale and move slider until 7.5 on the C scale comes under the hairline. The intermediate result, 4, appears under the right-hand index of the C scale.

12 	SLIDE RULE INSTRUCTIONS

We do not need to write it down but merely note it by moving the indicator until the hair-line is over the right-hand index of the C scale. Now we want to multiply this result by 2, the next factor in the numerator. Since two is out beyond the body of the rule, transfer the slider till the other (left-hand) index of the C scale is under the hair-line, and then move the indicator to 2 on the C scale. Thus, successive division and multiplication is continued until all the factors have been used. The order in which the factors are taken does not affect the result. With a little practice you will learn to take them in the order which will require the fewest settings. The following examples are for practice:

Example 56: (6/3.5) * (4/5) * (3.5/2.4) * (2.8/7) = .8

Example 57: 352 * (273/254) * (760/768) = 374

An alternative method of doing these examples is to proceed exactly as though you were multiplying all the factors together, except that whenever you come to a number in the denominator you use the CI scale instead of the C scale. The reader is advised to practice both methods and use whichever one he likes best.

3. The area of a circle. The area of a circle is found by multiplying 3.1416=PI by the square of the radius or by one-quarter the square of the diameter

Formula: A = PI * square( R )

A = PI * ( square( D ) / 4 )

Example 58: The radius of a circle is 0.25 inches; find its area.

SLIDE RULE INSTRUCTIONS 	13

Area = PI * square(0.25) = 0.196 square inches.

Set left-hand index of C scale over 0.25 on D scale. square(0.25) now appears above the left-hand index of the B scale. This can be multiplied by PI by moving the indicator to PI on the B scale and reading the answer .196 on the A scale. This is an example where it is convenient to multiply with the A and B scales.

Example 59: The diameter of a circle is 8.1 feet. What is its area?

Area = (PI / 4) * square(8.1)

= .7854 * square(8.1)

= 51.7 sq. inches.

Set right-hand index of the C scale over 8.1 on the D scale. Move the indicator till hair-line is over .7854 (the special long mark near 8) at the right hand of the B scale. Read the answer under the hair-line on the A scale. Another way of finding the area of a circle is to set 7854 on the B scale to one of the indices of the A scale, and read the area from the B scale directly above the given diameter on the D scale.

4. The circumference of a circle. Set the index of the B scale to the diameter and read the answer on the A scale opposite PI on the B scale

Formula: C = PI * D

C = 2 * PI * R

Example 60: The diameter of a circle is 1.54 inches, what is its circumference?

Set the left-hand index of the B scale to 1.54 on the A scale. Read the circumference 4.85 inches above PI on the B scale.

EXAMPLES FOR PRACTICE

61: What is the area of a circle 32-1/2 inches in diameter?

Answer 830 sq. inches

14 	SLIDE RULE INSTRUCTIONS

62: What is the area of a circle 24 inches in diameter?

Answer 452 sq. inches

63: What is the circumference of a circle whose diameter is 95 feet?

Answer 298 ft.

64: What is the circumference of a circle whose diameter is 3.65 inches?

Answer 11.5 inches

5. Ratio and Proportion.

Example 65:   3 : 7 : : 4 : X

or

(3/7) = (4/x)

Find X

Set 3 on C scale over 7 on D scale. Read X on D scale under 4 on C scale. In fact, any number on the C scale is to the number directly under it on the D scale as 3 is to 7.

PRACTICAL PROBLEMS SOLVED BY SLIDE RULE

1. Discount.

A firm buys a typewriter with a list price of $150, subject to a discount of 20% and 10%. How much does it pay?

A discount of 20% means 0.8 of the list price, and 10% more means

0.8 X 0.9 X 150 = 108.

To do this on the slide rule, put the index of the C scale opposite 8 on the D scale and move the indicator to 9 on the C scale. Then move the slider till the right-hand index of the C scale is under the hairline. Now, move the indicator to 150 on the C scale and read the answer $108 on the D scale. Notice that in this, as in many practical problems, there is no question about where the decimal point should go.

2. Sales Tax.

A man buys an article worth $12 and he must pay a sales tax of 1.5%. How much does he pay? A tax of 1.5% means he must pay 1.015 * 12.00.

SLIDE RULE INSTRUCTIONS 	15

Set index of C scale at 1.015 on D scale. Move indicator to 12 on C scale and read the answer $12.18 on the D scale.

A longer but more accurate way is to multiply 12 * .015 and add the result to $12.

3. Unit Price.

A motorist buys 17 gallons of gas at 19.5 cents per gallon. How much does he pay?

Set index of C scale at 17 on D scale and move indicator to 19.5 on C scale and read the answer $3.32 on the D scale.

4. Gasoline Mileage.

An automobile goes 175 miles on 12 gallons of gas.

What is the average gasoline consumption?

Set indicator over 175 on D scale and move slider till 12 is under hair-line. Read the answer 14.6 miles per gallon on the D scale under the left-hand index of the C scale.

5. Average Speed.

A motorist makes a trip of 256 miles in 7.5 hours. What is his average speed?

Set indicator over 256 on D scale. Move slider till 7.5 on the C scale is under the hair-line. Read the answer 34.2 miles per hour under the right-hand index of the C scale.

6. Decimal Parts of an Inch.

What is 5/16 of an inch expressed as decimal fraction?

Set 16 on C scale over 5 on D scale and read the result .3125 inches on the D scale under the left-hand index of the C scale.

7. Physics.

A certain quantity of gas occupies 1200 cubic centimeters at a temperature of 15 degrees C and 740 millimeters pressure. What volume does it occupy at 0 degrees C and 760 millimeters pressure?

16 	SLIDE RULE INSTRUCTIONS

Volume = 1200 X (740/760) * (273/288) = 1100 cubic cm.

Set 760 on C scale over 12 on D scale. Move indicator to 740 on C scale. Move slider till 288 on C scale is under hair-line. Move indicator to 273 on C scale. Read answer, 1110, under hair-line on D scale.

8. Chemistry.

How many grams of hydrogen are formed when 80 grams of zinc react with sufficient hydrochloric acid to dissolve the metal?

(80 / X ) = ( 65.4 / 2.01)

Set 65.4 on C scale over 2.01 on D scale. Read X = 2.46 grams under 80 on C scale.

In conclusion, we want to impress upon those to whom the slide rule is a new method of doing their mathematical calculations, and also the experienced operator of a slide rule, that if they will form a habit of, and apply themselves to, using a slide rule at work, study, or during recreations, they will be well rewarded in the saving of time and energy. ALWAYS HAVE YOUR SLIDE RULE AND INSTRUCTION BOOK WITH YOU, the same as you would a fountain pen or pencil.

The present day wonders of the twentieth century prove that there is no end to what an individual can accomplish--the same applies to the slide rule.

You will find after practice that you will be able to do many specialized problems that are not outlined in this instruction book. It depends entirely upon your ability to do what we advocate and to be slide-rule conscious in all your mathematical problems.

CONVERSION FACTORS

1. Length

1 mile = 5280 feet =1760 yards

1 inch = 2.54 centimeters

1 meter = 39.37 inches

2. Weight (or Mass)

1 pound = 16 ounces = 0.4536 kilograms

1 kilogram = 2.2 pounds

1 long ton = 2240 pounds

1 short ton = 2000 pounds

3. Volume

1 liquid quart = 0.945 litres

1 litre = 1.06 liquid quarts

1 U. S. gallon = 4 quarts = 231 cubic inches

4. Angular Measure

3.14 radians = PI radians = 180 degrees

1 radian = 57.30 degrees

5. Pressure

760 millimeters of mercury = 14.7 pounds per square inch

6. Power

1 horse power = 550 foot pounds per second = 746 watts

7. Miscellaneous

60 miles per hour = 88 feet per second

980 centimeters per second per second

= 32.2 feet per second per second

= acceleration of gravity.

1 cubic foot of water weighs 62.4 pounds

1 gallon of water weighs 8.34 pounds

Printed in U. S. A.

INSTRUCTIONS FOR USING A SLIDE RULE

COPYRIGHTED BY W. STANLEY & CO.

Commercial Trust Building, Philadelphia, Pa.

End of Project Gutenberg's Instruction for Using a Slide Rule, by W. Stanley

*** END OF THIS PROJECT GUTENBERG EBOOK INSTRUCTION FOR USING A SLIDE RULE ***

***** This file should be named 20214-doc.doc or 20214-doc.zip *****

This and all associated files of various formats will be found in:

http://www.gutenberg.org/2/0/2/1/20214/

Produced by Don Kostuch

Updated editions will replace the previous one--the old editions

will be renamed.

Creating the works from public domain print editions means that no

one owns a United States copyright in these works, so the Foundation

(and you!) can copy and distribute it in the United States without

permission and without paying copyright royalties.  Special rules,

set forth in the General Terms of Use part of this license, apply to

copying and distributing Project Gutenberg-tm electronic works to

protect the PROJECT GUTENBERG-tm concept and trademark.  Project

Gutenberg is a registered trademark, and may not be used if you

charge for the eBooks, unless you receive specific permission.  If you

do not charge anything for copies of this eBook, complying with the

rules is very easy.  You may use this eBook for nearly any purpose

such as creation of derivative works, reports, performances and

research.  They may be modified and printed and given away--you may do

practically ANYTHING with public domain eBooks.  Redistribution is

subject to the trademark license, especially commercial

redistribution.

*** START: FULL LICENSE ***

THE FULL PROJECT GUTENBERG LICENSE

PLEASE READ THIS BEFORE YOU DISTRIBUTE OR USE THIS WORK

To protect the Project Gutenberg-tm mission of promoting the free

distribution of electronic works, by using or distributing this work

(or any other work associated in any way with the phrase "Project

Gutenberg"), you agree to comply with all the terms of the Full Project

Gutenberg-tm License (available with this file or online at

http://gutenberg.org/license).

Section 1.  General Terms of Use and Redistributing Project Gutenberg-tm

electronic works

1.A.  By reading or using any part of this Project Gutenberg-tm

electronic work, you indicate that you have read, understand, agree to

and accept all the terms of this license and intellectual property

(trademark/copyright) agreement.  If you do not agree to abide by all

the terms of this agreement, you must cease using and return or destroy

all copies of Project Gutenberg-tm electronic works in your possession.

If you paid a fee for obtaining a copy of or access to a Project

Gutenberg-tm electronic work and you do not agree to be bound by the

terms of this agreement, you may obtain a refund from the person or

entity to whom you paid the fee as set forth in paragraph 1.E.8.

1.B.  "Project Gutenberg" is a registered trademark.  It may only be

used on or associated in any way with an electronic work by people who

agree to be bound by the terms of this agreement.  There are a few

things that you can do with most Project Gutenberg-tm electronic works

even without complying with the full terms of this agreement.  See

paragraph 1.C below.  There are a lot of things you can do with Project

Gutenberg-tm electronic works if you follow the terms of this agreement

and help preserve free future access to Project Gutenberg-tm electronic

works.  See paragraph 1.E below.

1.C.  The Project Gutenberg Literary Archive Foundation ("the Foundation"

or PGLAF), owns a compilation copyright in the collection of Project

Gutenberg-tm electronic works.  Nearly all the individual works in the

collection are in the public domain in the United States.  If an

individual work is in the public domain in the United States and you are

located in the United States, we do not claim a right to prevent you from

copying, distributing, performing, displaying or creating derivative

works based on the work as long as all references to Project Gutenberg

are removed.  Of course, we hope that you will support the Project

Gutenberg-tm mission of promoting free access to electronic works by

freely sharing Project Gutenberg-tm works in compliance with the terms of

this agreement for keeping the Project Gutenberg-tm name associated with

the work.  You can easily comply with the terms of this agreement by

keeping this work in the same format with its attached full Project

Gutenberg-tm License when you share it without charge with others.

1.D.  The copyright laws of the place where you are located also govern

what you can do with this work.  Copyright laws in most countries are in

a constant state of change.  If you are outside the United States, check

the laws of your country in addition to the terms of this agreement

before downloading, copying, displaying, performing, distributing or

creating derivative works based on this work or any other Project

Gutenberg-tm work.  The Foundation makes no representations concerning

the copyright status of any work in any country outside the United

States.

1.E.  Unless you have removed all references to Project Gutenberg:

1.E.1.  The following sentence, with active links to, or other immediate

access to, the full Project Gutenberg-tm License must appear prominently

whenever any copy of a Project Gutenberg-tm work (any work on which the

phrase "Project Gutenberg" appears, or with which the phrase "Project

Gutenberg" is associated) is accessed, displayed, performed, viewed,

copied or distributed:

This eBook is for the use of anyone anywhere at no cost and with

almost no restrictions whatsoever.  You may copy it, give it away or

re-use it under the terms of the Project Gutenberg License included

with this eBook or online at www.gutenberg.org

1.E.2.  If an individual Project Gutenberg-tm electronic work is derived

from the public domain (does not contain a notice indicating that it is

posted with permission of the copyright holder), the work can be copied

and distributed to anyone in the United States without paying any fees

or charges.  If you are redistributing or providing access to a work

with the phrase "Project Gutenberg" associated with or appearing on the

work, you must comply either with the requirements of paragraphs 1.E.1

through 1.E.7 or obtain permission for the use of the work and the

Project Gutenberg-tm trademark as set forth in paragraphs 1.E.8 or

1.E.9.

1.E.3.  If an individual Project Gutenberg-tm electronic work is posted

with the permission of the copyright holder, your use and distribution

must comply with both paragraphs 1.E.1 through 1.E.7 and any additional

terms imposed by the copyright holder.  Additional terms will be linked

to the Project Gutenberg-tm License for all works posted with the

permission of the copyright holder found at the beginning of this work.

1.E.4.  Do not unlink or detach or remove the full Project Gutenberg-tm

License terms from this work, or any files containing a part of this

work or any other work associated with Project Gutenberg-tm.

1.E.5.  Do not copy, display, perform, distribute or redistribute this

electronic work, or any part of this electronic work, without

prominently displaying the sentence set forth in paragraph 1.E.1 with

active links or immediate access to the full terms of the Project

Gutenberg-tm License.

1.E.6.  You may convert to and distribute this work in any binary,

compressed, marked up, nonproprietary or proprietary form, including any

word processing or hypertext form.  However, if you provide access to or

distribute copies of a Project Gutenberg-tm work in a format other than

"Plain Vanilla ASCII" or other format used in the official version

posted on the official Project Gutenberg-tm web site (www.gutenberg.org),

you must, at no additional cost, fee or expense to the user, provide a

copy, a means of exporting a copy, or a means of obtaining a copy upon

request, of the work in its original "Plain Vanilla ASCII" or other

form.  Any alternate format must include the full Project Gutenberg-tm

License as specified in paragraph 1.E.1.

1.E.7.  Do not charge a fee for access to, viewing, displaying,

performing, copying or distributing any Project Gutenberg-tm works

unless you comply with paragraph 1.E.8 or 1.E.9.

1.E.8.  You may charge a reasonable fee for copies of or providing

access to or distributing Project Gutenberg-tm electronic works provided

that

- You pay a royalty fee of 20% of the gross profits you derive from

the use of Project Gutenberg-tm works calculated using the method

you already use to calculate your applicable taxes.  The fee is

owed to the owner of the Project Gutenberg-tm trademark, but he

has agreed to donate royalties under this paragraph to the

Project Gutenberg Literary Archive Foundation.  Royalty payments

must be paid within 60 days following each date on which you

prepare (or are legally required to prepare) your periodic tax

returns.  Royalty payments should be clearly marked as such and

sent to the Project Gutenberg Literary Archive Foundation at the

address specified in Section 4, "Information about donations to

the Project Gutenberg Literary Archive Foundation."

- You provide a full refund of any money paid by a user who notifies

you in writing (or by e-mail) within 30 days of receipt that s/he

does not agree to the terms of the full Project Gutenberg-tm

License.  You must require such a user to return or

destroy all copies of the works possessed in a physical medium

and discontinue all use of and all access to other copies of

Project Gutenberg-tm works.

- You provide, in accordance with paragraph 1.F.3, a full refund of any

money paid for a work or a replacement copy, if a defect in the

electronic work is discovered and reported to you within 90 days

of receipt of the work.

- You comply with all other terms of this agreement for free

distribution of Project Gutenberg-tm works.

1.E.9.  If you wish to charge a fee or distribute a Project Gutenberg-tm

electronic work or group of works on different terms than are set

forth in this agreement, you must obtain permission in writing from

both the Project Gutenberg Literary Archive Foundation and Michael

Hart, the owner of the Project Gutenberg-tm trademark.  Contact the

Foundation as set forth in Section 3 below.

1.F.

1.F.1.  Project Gutenberg volunteers and employees expend considerable

effort to identify, do copyright research on, transcribe and proofread

public domain works in creating the Project Gutenberg-tm

collection.  Despite these efforts, Project Gutenberg-tm electronic

works, and the medium on which they may be stored, may contain

"Defects," such as, but not limited to, incomplete, inaccurate or

corrupt data, transcription errors, a copyright or other intellectual

property infringement, a defective or damaged disk or other medium, a

computer virus, or computer codes that damage or cannot be read by

your equipment.

1.F.2.  LIMITED WARRANTY, DISCLAIMER OF DAMAGES - Except for the "Right

of Replacement or Refund" described in paragraph 1.F.3, the Project

Gutenberg Literary Archive Foundation, the owner of the Project

Gutenberg-tm trademark, and any other party distributing a Project

Gutenberg-tm electronic work under this agreement, disclaim all

liability to you for damages, costs and expenses, including legal

fees.  YOU AGREE THAT YOU HAVE NO REMEDIES FOR NEGLIGENCE, STRICT

LIABILITY, BREACH OF WARRANTY OR BREACH OF CONTRACT EXCEPT THOSE

PROVIDED IN PARAGRAPH F3.  YOU AGREE THAT THE FOUNDATION, THE

TRADEMARK OWNER, AND ANY DISTRIBUTOR UNDER THIS AGREEMENT WILL NOT BE

LIABLE TO YOU FOR ACTUAL, DIRECT, INDIRECT, CONSEQUENTIAL, PUNITIVE OR

INCIDENTAL DAMAGES EVEN IF YOU GIVE NOTICE OF THE POSSIBILITY OF SUCH

DAMAGE.

1.F.3.  LIMITED RIGHT OF REPLACEMENT OR REFUND - If you discover a

defect in this electronic work within 90 days of receiving it, you can

receive a refund of the money (if any) you paid for it by sending a

written explanation to the person you received the work from.  If you

received the work on a physical medium, you must return the medium with

your written explanation.  The person or entity that provided you with

the defective work may elect to provide a replacement copy in lieu of a

refund.  If you received the work electronically, the person or entity

providing it to you may choose to give you a second opportunity to

receive the work electronically in lieu of a refund.  If the second copy

is also defective, you may demand a refund in writing without further

opportunities to fix the problem.

1.F.4.  Except for the limited right of replacement or refund set forth

in paragraph 1.F.3, this work is provided to you 'AS-IS' WITH NO OTHER

WARRANTIES OF ANY KIND, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO

WARRANTIES OF MERCHANTIBILITY OR FITNESS FOR ANY PURPOSE.

1.F.5.  Some states do not allow disclaimers of certain implied

warranties or the exclusion or limitation of certain types of damages.

If any disclaimer or limitation set forth in this agreement violates the

law of the state applicable to this agreement, the agreement shall be

interpreted to make the maximum disclaimer or limitation permitted by

the applicable state law.  The invalidity or unenforceability of any

provision of this agreement shall not void the remaining provisions.

1.F.6.  INDEMNITY - You agree to indemnify and hold the Foundation, the

trademark owner, any agent or employee of the Foundation, anyone

providing copies of Project Gutenberg-tm electronic works in accordance

with this agreement, and any volunteers associated with the production,

promotion and distribution of Project Gutenberg-tm electronic works,

harmless from all liability, costs and expenses, including legal fees,

that arise directly or indirectly from any of the following which you do

or cause to occur: (a) distribution of this or any Project Gutenberg-tm

work, (b) alteration, modification, or additions or deletions to any

Project Gutenberg-tm work, and (c) any Defect you cause.

Section  2.  Information about the Mission of Project Gutenberg-tm

Project Gutenberg-tm is synonymous with the free distribution of

electronic works in formats readable by the widest variety of computers

including obsolete, old, middle-aged and new computers.  It exists

because of the efforts of hundreds of volunteers and donations from

people in all walks of life.

Volunteers and financial support to provide volunteers with the

assistance they need, is critical to reaching Project Gutenberg-tm's

goals and ensuring that the Project Gutenberg-tm collection will

remain freely available for generations to come.  In 2001, the Project

Gutenberg Literary Archive Foundation was created to provide a secure

and permanent future for Project Gutenberg-tm and future generations.

To learn more about the Project Gutenberg Literary Archive Foundation

and how your efforts and donations can help, see Sections 3 and 4

and the Foundation web page at http://www.pglaf.org.

Section 3.  Information about the Project Gutenberg Literary Archive

Foundation

The Project Gutenberg Literary Archive Foundation is a non profit

501(c)(3) educational corporation organized under the laws of the

state of Mississippi and granted tax exempt status by the Internal

Revenue Service.  The Foundation's EIN or federal tax identification

number is 64-6221541.  Its 501(c)(3) letter is posted at

http://pglaf.org/fundraising.  Contributions to the Project Gutenberg

Literary Archive Foundation are tax deductible to the full extent

permitted by U.S. federal laws and your state's laws.

The Foundation's principal office is located at 4557 Melan Dr. S.

Fairbanks, AK, 99712., but its volunteers and employees are scattered

throughout numerous locations.  Its business office is located at

809 North 1500 West, Salt Lake City, UT 84116, (801) 596-1887, email

business@pglaf.org.  Email contact links and up to date contact

information can be found at the Foundation's web site and official

page at http://pglaf.org

For additional contact information:

Dr. Gregory B. Newby

Chief Executive and Director

gbnewby@pglaf.org

Section 4.  Information about Donations to the Project Gutenberg

Literary Archive Foundation

Project Gutenberg-tm depends upon and cannot survive without wide

spread public support and donations to carry out its mission of

increasing the number of public domain and licensed works that can be

freely distributed in machine readable form accessible by the widest

array of equipment including outdated equipment.  Many small donations

($1 to $5,000) are particularly important to maintaining tax exempt

status with the IRS.

The Foundation is committed to complying with the laws regulating

charities and charitable donations in all 50 states of the United

States.  Compliance requirements are not uniform and it takes a

considerable effort, much paperwork and many fees to meet and keep up

with these requirements.  We do not solicit donations in locations

where we have not received written confirmation of compliance.  To

SEND DONATIONS or determine the status of compliance for any

particular state visit http://pglaf.org

While we cannot and do not solicit contributions from states where we

have not met the solicitation requirements, we know of no prohibition

against accepting unsolicited donations from donors in such states who

approach us with offers to donate.

International donations are gratefully accepted, but we cannot make

any statements concerning tax treatment of donations received from

outside the United States.  U.S. laws alone swamp our small staff.

Please check the Project Gutenberg Web pages for current donation

methods and addresses.  Donations are accepted in a number of other

ways including checks, online payments and credit card donations.

To donate, please visit: http://pglaf.org/donate

Section 5.  General Information About Project Gutenberg-tm electronic

works.

Professor Michael S. Hart is the originator of the Project Gutenberg-tm

concept of a library of electronic works that could be freely shared

with anyone.  For thirty years, he produced and distributed Project

Gutenberg-tm eBooks with only a loose network of volunteer support.

Project Gutenberg-tm eBooks are often created from several printed

editions, all of which are confirmed as Public Domain in the U.S.

unless a copyright notice is included.  Thus, we do not necessarily

keep eBooks in compliance with any particular paper edition.

Most people start at our Web site which has the main PG search facility:

http://www.gutenberg.org

This Web site includes information about Project Gutenberg-tm,

including how to make donations to the Project Gutenberg Literary

Archive Foundation, how to help produce our new eBooks, and how to

subscribe to our email newsletter to hear about new eBooks.